OpenCV-Based Food Quality

Inspection System

A working Python implementation, explained end-to-end

1. What is this project?

This project builds an automated food quality inspection system using OpenCV (computer vision) and machine learning. Instead of a human visually checking each apple, tomato, batch of rice, or loaf of bread for defects, a camera and an algorithm do it — consistently, in milliseconds, at conveyor-belt speed.

The system takes a photo of a food item, cleans up the image, isolates the item from the background, measures its color/texture/shape, and then a trained classifier decides: is this item GOOD or DEFECTIVE? Based on that decision, a signal is sent to physically accept or reject the item from the production line.

## Why does this matter?

- Manual inspection is slow, inconsistent, and tiring — different inspectors (or the same inspector at different times) can grade the same item differently.

- It doesn't scale — a packing line processing thousands of items per hour cannot be checked item-by-item by hand.

- Computer vision is non-contact, doesn't damage the food, and can run 24/7 without fatigue.

# 2. Real-world use cases

This is not just an academic exercise — this exact category of system (machine vision + ML defect detection) is already used across the food industry:

- Fruit & vegetable packing houses — apples, tomatoes, citrus, potatoes are graded by size, color, and surface defects (bruising, rot, cracking) before packing/export.

- Rice and grain mills — separating broken grains, discolored grains, husks, and foreign matter (stones, insects) from good grain at high speed.

- Bakeries and food manufacturing — checking loaf shape, burn spots, or mold before packaging.

- Meat, poultry, and seafood plants — detecting bruising, discoloration, or contamination.

- Export quality certification — grading produce into Grade A / B / C tiers, since buyers pay different prices for different visual quality.

The advantage over manual inspection: consistency (no subjective judgment or fatigue), speed (real-time, thousands of items/hour), and scalability (one camera + algorithm replaces a full shift of human inspectors).

# 3. How it works — system architecture

The system is built as a six-stage pipeline. Each stage takes the output of the previous stage and refines it further, until the final stage produces an accept/reject decision.

Figure 1: The six-stage inspection pipeline, from image capture to sorting decision.

| Stage | Function | Key OpenCV / tooling |
| --- | --- | --- |
| 1. Image Acquisition | Capture sample images under controlled illumination | cv2.VideoCapture, camera calibration |
| 2. Pre-processing | Noise removal, color correction, background removal | cv2.GaussianBlur, CLAHE, cv2.threshold |
| 3. Segmentation | Isolate the food item from background/packaging | cv2.findContours, cv2.Canny |
| 4. Feature Extraction | Quantify color, texture, size, shape, defects | cv2.calcHist, GLCM, Hu Moments |
| 5. Classification | Assign a quality grade / defect label | scikit-learn Random Forest |
| 6. Decision & Actuation | Trigger sorting mechanism / reject signal | Timed inference loop, actuator signal |

# 4. Code walkthrough

The full implementation lives in one file, food_quality_inspection.py. Since there's no photo dataset available in this environment, the script generates a synthetic-but-realistic dataset in code — the pipeline logic itself is identical to what you'd run on real camera photos.

## 4.1 Stage 1 — Image acquisition

acquire_image() simulates a camera capturing a food item on a conveyor belt. It adds realistic imperfections: uneven lighting across the frame (_lighting_jitter) and sensor noise (_add_sensor_noise), so the pipeline has to be robust — just like a real camera feed.

def acquire_image(food_type, defective):
    img = _GENERATORS[food_type](defective)
    img = _lighting_jitter(img, strength=RNG.uniform(0.05, 0.2))
    img = _add_sensor_noise(img, sigma=RNG.uniform(4, 10))
    return img

## 4.2 Stage 2 — Pre-processing

preprocess() blurs out sensor noise, applies CLAHE (a contrast-correction technique) so lighting differences don't fool the color analysis, then uses Otsu thresholding to separate the food item from the background.

def preprocess(img):
    denoised = cv2.GaussianBlur(img, (5, 5), 0)
    ...
    clahe = cv2.createCLAHE(clipLimit=2.0, tileGridSize=(8, 8))
    ...
    _, fg_mask = cv2.threshold(gray, 0, 255,
                               cv2.THRESH_BINARY_INV + cv2.THRESH_OTSU)
    return corrected, fg_mask, foreground

## 4.3 Stage 3 — Segmentation

segment() finds the outline(s) of the food item using cv2.findContours. This step matters most for rice, where many grains can touch each other in one frame — the system must separate each grain, not just find one big blob.

def segment(fg_mask):
    edges = cv2.Canny(fg_mask, 50, 150)
    contours, _ = cv2.findContours(fg_mask, cv2.RETR_EXTERNAL,
                                    cv2.CHAIN_APPROX_SIMPLE)
    contours = [c for c in contours if cv2.contourArea(c) > 15]
    return edges, contours

## 4.4 Stage 4 — Feature extraction

extract_features() converts pixels into numbers the classifier can learn from: an HSV color histogram (what colors are present), GLCM texture statistics (how rough/smooth the surface is), and shape descriptors (contour area, perimeter, Hu Moments).

def extract_features(corrected, fg_mask, contours):
    hsv = cv2.cvtColor(corrected, cv2.COLOR_BGR2HSV)
    hist = cv2.calcHist([hsv], [0], fg_mask, [16], [0, 180])
    glcm = graycomatrix(gray_masked, distances=[3], angles=[0], ...)
    contrast = graycoprops(glcm, 'contrast')[0, 0]
    hu_log = -np.sign(hu) * np.log10(np.abs(hu) + 1e-30)
    return np.concatenate([hist, [contrast, ...], hu_log])

## 4.5 Stage 5 & 6 — Classification and decision

train_and_evaluate() trains one Random Forest classifier per food type on the extracted features, then times how long a single prediction takes — this stands in for the moment a real system would fire a reject signal to a sorting arm.

clf = RandomForestClassifier(n_estimators=200, max_depth=8)
clf.fit(X_train, y_train)

t0 = time.perf_counter()
prediction = clf.predict(x.reshape(1, -1))[0]
elapsed = time.perf_counter() - t0   # this is the 'decision speed'

# 5. Real-time example: one item through the pipeline

Here's what happens to a single defective apple as it moves through all six stages, left to right: the raw camera capture, the cleaned-up image, the background-removal mask, the detected outline, and finally the reject decision.

Figure 2: All six stages applied to a defective sample of each food type.

Reading this like a real inspector would: the camera captures the apple (column 1), the software cleans up lighting and removes the gray conveyor background (columns 2-3), draws a green outline around exactly where the apple is (column 4), and — because the extracted features didn't match a healthy apple's expected color/texture — the system flags it for rejection (column 5), all within about 10 milliseconds.

## The dataset used

Four food types were tested, each with 'good' and 'defective' examples covering the defect types real inspection lines look for: bruising (apple), cracking/discoloration (tomato), broken grains/foreign matter (rice), and burn spots/mold (bread).

Figure 3: Sample images from the dataset — two good, two defective, per food type.

# 6. Results

Each food type's classifier was evaluated on held-out test samples it had never seen during training. The chart below reports defect-detection accuracy and how long a single decision takes.

Figure 4: Accuracy and per-sample inference latency by food type.

The confusion matrices below break this down further — how many good items were correctly passed, how many defective items were correctly caught, and where mistakes happened:

Figure 5: Confusion matrices per food type (rows = true label, columns = predicted label).

| Food | Defect type detected | Accuracy | Inference time |
| --- | --- | --- | --- |
| Apple | Surface decay / bruising | ~83% | ~10 ms |
| Tomato | Cracking / discoloration | ~83% | ~10 ms |
| Rice | Broken grains / foreign matter | ~94% | ~10 ms |
| Bread | Burn spots / mold | ~94% | ~10 ms |

For comparison, real inspection-line systems typically report similar accuracy in the high-80s to high-90s percent range and inference times under 50 milliseconds — which is exactly the ballpark needed for real-time sorting on a moving conveyor.

# 7. Conclusion

This project demonstrates that a full food-inspection pipeline — camera capture, pre-processing, segmentation, feature extraction, classification, and a sort decision — can be built with open-source tools (OpenCV, scikit-learn, scikit-image) and run fast enough for real production use.

## Possible future improvements

- Deep learning (CNNs) for texture-heavy defects that simple color/shape features miss.

- Hyperspectral or near-infrared imaging to catch defects invisible to the human eye or a normal camera.

- Running the model on small edge devices (e.g. Raspberry Pi + camera) directly at the point of packing or preparation.

- A larger, real-world labeled photo dataset covering more food types and more lighting conditions.

One-sentence summary to remember: this system turns "does this food look okay?" — a subjective human judgment — into "are these measured numbers inside the expected range?" — a fast, consistent, machine-checkable decision.